# VLSID R23 SHORT ANSWER QUESTIONS WITH ANSWERS

## UNIT 1

- 1. What is MOS technology?

Answer: MOS technology uses Metal–Oxide–Semiconductor structures to form transistors used in integrated circuits.

- 2. Define PMOS and NMOS technologies.

Answer: PMOS uses p-channel MOSFETs, whereas NMOS uses n-channel MOSFETs for logic implementation.

- 3. What are the advantages of CMOS over NMOS technology?

Answer: CMOS consumes low static power, has high noise immunity, and provides high packing density.

- 4. What is Bi-CMOS technology?

Answer: A combination of Bipolar and CMOS technologies on the same chip.

- 5. Why is CMOS preferred in modern VLSI circuits?

Answer: Due to its low power consumption, high performance, and excellent scalability.

- 6. Define threshold voltage (VT).

Answer: The gate voltage required to create a conducting channel in the MOSFET.

- 7. What is transconductance (gm)?

Answer: The change in drain current with respect to gate voltage, gm = ∂ID/∂VGS.

- 8. What is a pass transistor?

Answer: A MOS transistor used as a switch to pass logic levels between nodes.

- 9. Why does an NMOS pass transistor pass a weak ‘1’?

Answer: Because it cannot pass a voltage higher than VDD − VT.

- 10. What is a transmission gate?

Answer: A parallel combination of NMOS and PMOS that passes both logic '0' and '1' strongly.

## UNIT 2

- 1. What is physical design?

Answer: Conversion of a circuit netlist into a geometric layout.

- 2. What is floor planning?

Answer: Arrangement of major functional blocks on a chip.

- 3. What is placement and routing?

Answer: Placement arranges cells; routing connects the cells with metal wires.

- 4. What is design verification?

Answer: Checking if the design meets functional and physical specifications.

- 5. What is the purpose of polysilicon?

Answer: It is used mainly for MOS transistor gates.

- 6. What is a stick diagram?

Answer: A simplified layout representation using colored lines.

- 7. Why are stick diagrams used?

Answer: They help in early visualization of layout structure.

- 8. What are design rules?

Answer: Geometric constraints that ensure correct fabrication.

- 9. Why are design rules important?

Answer: They prevent manufacturing defects and ensure reliability.

- 10. What is MOS scaling?

Answer: Reducing device dimensions and voltages for better performance.

## UNIT 3

- 1. What is a logic gate?

Answer: A basic digital circuit that performs logic operations.

- 2. Define complex gate.

Answer: A gate that combines multiple logic functions in one structure.

- 3. What is switch logic?

Answer: Logic implementation using MOS transistors as switches.

- 4. What is sheet resistance (Rs)?

Answer: Resistance of a thin film measured in ohms per square.

- 5. Formula for sheet resistance.

Answer: Rs = ρ / t.

- 6. Effect of sheet resistance.

Answer: Higher Rs increases delay and slows circuit performance.

- 7. What is fan-out?

Answer: Number of inputs driven by one output.

- 8. What is area capacitance?

Answer: Capacitance formed due to overlapping conductive layers.

- 9. What is overlap capacitance?

Answer: Capacitance between gate and source/drain due to gate overlap.

- 10. Components of wiring capacitance.

Answer: Area, fringe, and coupling capacitances.

## UNIT 4

- 1. What is a shifter?

Answer: A circuit that shifts binary data left or right.

- 2. What is a barrel shifter?

Answer: A fast shifter that performs multi-bit shifts in one cycle.

- 3. What is an array multiplier?

Answer: A multiplier built using an array of adders.

- 4. What is Booth’s algorithm?

Answer: An algorithm that reduces the number of partial products.

- 5. What is a synchronous counter?

Answer: A counter where all flip-flops share the same clock.

- 6. Define standard cell design.

Answer: A semi-custom technique using pre-designed logic cells.

- 7. What is semi-custom design?

Answer: A design approach combining fixed and customizable blocks.

- 8. What is dynamic power?

Answer: Power consumed during switching activity.

- 9. What is voltage scaling?

Answer: Reducing supply voltage to lower power consumption.

- 10. What is a parity generator?

Answer: A circuit that produces parity bits for error detection.

## UNIT 5

- 1. Why test CMOS circuits?

Answer: To detect manufacturing defects and ensure reliable operation.

- 2. Define DFT.

Answer: Design for Testability—making circuits easier to test.

- 3. What is BIST?

Answer: A built-in mechanism for self-testing inside ICs.

- 4. Advantages of BIST.

Answer: Low cost, high speed, and improved fault coverage.

- 5. What is a test pattern generator?

Answer: A circuit (like an LFSR) that generates test patterns.

- 6. What is a signature analyzer?

Answer: A circuit that compresses test outputs into a signature.

- 7. What is ATPG?

Answer: Automatic Test Pattern Generation for fault detection.

- 8. What are stuck-at faults?

Answer: Faults that fix a signal at logic 0 or 1.

- 9. What are bridging faults?

Answer: Unintended shorts between two signal lines.

- 10. Why is sequential testing harder?

Answer: Because outputs depend on present and past states.