UNIT – I (8086 ARCHITECTURE)

Q1. Define pipelining?
Ans: Pipelining is a technique where multiple instruction stages (fetch, decode, execute) are overlapped to improve execution speed and overall throughput.

Q2. Define microprocessor?
Ans: A microprocessor is a programmable multipurpose IC that performs arithmetic, logic, and control operations of a computer system.

Q3. What is the need for ALE signal?
Ans: ALE (Address Latch Enable) is used to demultiplex the lower 16 address/data lines (AD0–AD15) to separate the address from data in 8086.

Q4. What is the format of flag register in 8086 microprocessor?
Ans: 8086 has a 16-bit flag register:

- Status flags: CF, PF, AF, ZF, SF, OF

- Control flags: TF, IF, DF

Q5. What is memory segmentation?
Ans: Memory segmentation is dividing the memory into logical segments (code, data, stack, extra) to allow modular programming and efficient memory use.

Q6. What is meant by physical addressing in 8086?
Ans: Physical address = (Segment × 10) + Offset.
It is the actual address in memory accessed by the processor.

Q7. What is MN/MX’?
Ans: MN/MX’ pin selects Minimum mode (single processor system) or Maximum mode (multiprocessor system).

Q8. List the functional units available in 8086 internal architecture?
Ans:

- Bus Interface Unit (BIU)

- Execution Unit (EU)

Q9. Write the special functions carried by the general-purpose registers of 8086.
Ans:

- AX → Accumulator (arithmetic/logic)

- BX → Base register (indexed addressing)

- CX → Counter (loop, shift/rotate)

- DX → Data register (multiplication/division, I/O operations)

Q10. What are the advantages of memory segmentation?
Ans:

- Modular programming

- Better memory management

- Larger memory access (1 MB) with 16-bit addresses

- Code and data protection

UNIT – II (PROGRAMMING WITH 8086)

Q11. List the various addressing modes present in 8086?
Ans: Immediate, Register, Direct, Register Indirect, Based, Indexed, Based-indexed, Relative, Implied.

Q12. What are macros & procedures?
Ans:

- Macro: A group of instructions given a name, expanded at assembly time.

- Procedure: A block of code executed when called, supports modularity and reusability.

Q13. What are called assembler directives? Give two examples.
Ans: Directives are commands to the assembler, not executed by CPU.
Examples: DB (define byte), ORG (origin).

Q14. Write an ALP for 8086 to perform addition of two 8-bit unsigned numbers.

MOV AL, 25H

ADD AL, 15H

MOV BL, AL

HLT

Q15. List string instructions.
Ans: The string instructions are

- MOVS → Move String

- CMPS → Compare String

- SCAS → Scan String

- LODS → Load String

- STOS → Store String

Q16. What is an assembler?
Ans: Assembler is a software tool that converts assembly language programs into machine code.

Q17. List any four arithmetic instructions in 8086 microprocessor.
Ans:  . List of arithmetic instructions in 8086 microprocessor ADD, SUB, INC, DEC, MUL, DIV, Signed MUL,DIV,UN Signed MUL,DIV

Q18. What are the Addressing Modes for sequential control flow instructions in 8086?
Ans: The Addressing Modes for sequential control flow instructions in 8086 Direct Addressing Mode, Immediate Addressing Mode, Register Addressing Mode, Register indirect  Addressing Mode.

Q19. What are the Addressing Modes for control transfer instructions in 8086?
Ans: The addressing modes for control transfer Instruction in 8086 Direct Addressing Mode, Indirect Addressing Mode, Relative Addressing Mode (short/near/far).

Q20. What will be the content of Stack Pointer (SP) after PUSH operation and after POP operation?
Ans: The content of Stack Pointer (SP) after PUSH operation and after POP operation

- PUSH → SP = SP – 2

- POP → SP = SP + 2

UNIT – III (INTERFACING & PERIPHERALS)

Q21. Define Baud rate.
Baud rate is the number of signal changes (symbols) per second, deciding data transfer speed.

Q22. What is bit set reset (BSR) mode of 8255?
BSR mode allows individual bits of Port C to be set or cleared for control purposes.

Q23. What is asynchronous and synchronous transmission?
Asynchronous uses start/stop bits without a clock, while synchronous uses a common clock signal.

Q24. What is simplex and duplex transmission?
Simplex is one-way, half-duplex is two-way but one at a time, and full-duplex is two-way simultaneously.

Q25. What is serial and parallel transmission?
Serial sends data bit by bit, while parallel sends multiple bits together in one clock cycle.

Q26. Registers of 8255.
It has Port A, Port B, Port C, and a Control Register for operation settings.

Q27. Function of EOC and SC in ADC.
EOC indicates conversion completion, and SC starts the A/D conversion process.

Q28. What is a priority resolver?
It selects the highest priority request when multiple devices ask for service at once.

Q29. Registers of 8237 DMA controller.
It has DMA address, Terminal count, Command, Status, Mode, and Request registers.

Q30. What is MODEM?
A MODEM converts digital signals into analog (modulation) and back into digital (demodulation).

UNIT – IV (8051 MICROCONTROLLER)

Q31. PSW format of 8051.
The PSW is an 8-bit register with flags: CY, AC, F0, RS1, RS0, OV, –, and P.

Q32. Which port is used as address/data lines?
The Port 0 works as multiplexed address/data lines (P0.0–P0.7).

Q33. Microprocessor vs Microcontroller.
A microprocessor has only CPU and needs external parts, while a microcontroller has CPU, memory, and I/O built-in.

Q34. What is a special function register (SFR)?
SFRs are registers in 8051 reserved for control and status functions like PSW, TCON.

Q35. Function of RS0 and RS1 in PSW.
RS0 and RS1 bits select one of the four register banks (Bank 0–3).

Q36. Features of 8051.
It has an 8-bit CPU, 4 KB ROM, 128 B RAM, 4 I/O ports, 2 timers, and a serial port.

Q37. Instruction classification of 8051.
Instructions are grouped as data transfer, arithmetic, logical, branching, and bit manipulation.

Q38. Number of I/O ports in 8051.
8051 has 4 ports (P0–P3), providing 32 I/O lines in total.

Q39. Special functions of Port 3.
Port 3 pins serve as alternate functions such as RxD, TxD, INT0, INT1, T0, T1, WR, and RD.

Q40. Addressing modes of 8051.
It supports Immediate, Register, Direct, Indirect, and Indexed addressing modes.

UNIT – V (LOW POWER RISC MSP430)

Q41. Differentiate RISC and CISC.
RISC has simple, fixed-length fast instructions, while CISC has complex, variable-length slower instructions.

Q42. Define embedded systems.
An embedded system is a dedicated computer system designed for a specific real-time function.

Q43. Features of MSP430.
It is a 16-bit, ultra-low power RISC microcontroller with ADC/DAC, timers, and flexible clocks.

Q44. Harvard vs Von Neumann architectures.
Harvard uses separate program/data memory, while Von Neumann uses common memory for both.

Q45. What is program counter?
The program counter is a register that holds the address of the next instruction to execute.

Q46. Addressing modes of MSP430.
It supports Register, Indexed, Indirect, Auto-increment, Immediate, Absolute, and Symbolic modes.

Q47. Define MSP430 microcontroller.
MSP430 is a 16-bit RISC microcontroller by Texas Instruments, optimized for low-power embedded use.

Q48. Flags of status register in MSP430.
Flags are Carry (C), Zero (Z), Negative (N), and Overflow (V).

Q49. What is Watchdog timer (WDT)?
WDT resets the system if the program hangs, ensuring reliable system operation.

Q50. What is brownout circuit?
It resets or halts the microcontroller when supply voltage drops below a safe level.