NARAYANA ENGINEERING COLLEGE::NELLORE

(AUTONOMOUS)

Class : I B.TECH - ISEM Academic Year :2022-23 Course Title : Engineering Graphics Faculty : Dr. B V Krishnaiah Branch :Common to all Branches

MODULE-1

| S.NO | QUESTION BANK | CO | BL |  | MARKS |
| --- | --- | --- | --- | --- | --- |
| - 1 | a). Construct the Regular Pentagon of each side equal to 50mm b). Construct the Regular Hexagon of each side equal to 50mm | 1 | 1 |  | 7+7 |
| - 2 | Construct the Regular Polygons from Square to Octagon in single diagram, each side equal to 40mm. | 1 | 2 |  | 14 |
| - 3 | Construct an Ellipse when the distance between focus and the directrix is 50 mm and Eccentricity is 2/3. Draw a Tangent and Normal at any on the curve. | 1 | 2 |  | 14 |
| - 3 | Construct a Parabola when the distance between focus and the directrix is 60 mm. Draw a Tangent and Normal at 40 mm from directrix line. | 1 | 3 |  | 14 |
| - 4 | Construct a Hyperbola when the distance between focus and directrix is 60 mm, the Eccentricity is 4/3. Draw a Tangent and Normal at any point on the curve. | 1 | 3 |  | 14 |
| - 5 | A coin of 40 mm diameter rolls over a horizontal table without slipping. A point on the circumference of the coin is in contact with the table surface in the beginning and after one complete revolution. Draw the path traced by the point. Draw a tangent and a normal to the curve at any point on the curve. | 1 | 2 |  | 14 |
|  | Draw an Epi-Cycloid of rolling circle 40 mm diameter (2r) which rolls outside of another circle (Base circle) of 150mm diameter (2R) for one complete revolution. Draw a Tangent and Normal at any point on the curve. | 1 | 2 |  | 14 |
|  | Draw a hypo-cycloid of a circle for 40mm diameter, which rolls inside of another circle of 200 mm diameter for one complete revolution in counter clock-wise. Draw a tangent and normal at any point on the curve. | 1 | 2 |  | 14 |
|  | A coir is unwound from a dram of 40 mm diameter. Draw the locus of the free end of the coir for unwinding through an angle of 3600. Draw also a Normal and Tangent at any point on the curve. | 1 | 1 |  | 14 |
|  | Construct a plain scale to show in meters when 1 centimeter represents 4 meters and long enough to measure up to 50 meters. Find RF and mark on your scale a distance of 36 meters. | 1 | 3 |  | 14 |
|  | Construct a diagonal scale of RF= 1:32,00,000 to show kilometers and long enough to measure up to 400 km. Show the distance of 257 km and 333 km on your scale. | 1 | 3 |  | 14 |
|  | Construct a vernier scale to read meters, decimeters and centimeters and long enough to measure up to 4 meter. RF of the scale is 1/20. Mark onyour scale a distance of 2.28 meters. ( Backward/ Retrograde Vernier) | 1 | 3 |  | 14 |
| MODULE-2 |  |  |  |  |  |
| - 6 | Draw the projections of the following points on the same ground line, keeping the projectors 25 mm apart. - 40 mm above the HP and 20 mm behind the VP. - 40 mm above the HP and 25 mm in front of the VP. - In the VP and 40 mm above the HP. - 25 mm below the HP and 25 mm behind the VP. - 15 mm above the HP and 50 mm behind the VP. - 40 mm below the HP and 25 mm in front of the VP. - In both the HP and the VP. | 2 | 3 |  | 14 |
| - 7 | A Point ‘P’ is 35 mm above the HP and 50 mm in front of the VP. Another point Q is 25 mm behind the VP and 40 mm below the HP. Draw the projections of P and Q keeping the distance between their projections equal to 90 mm. Draw straight lines joining (i) their top views and ii) their front views. | 2 | 3 |  | 14 |
|  | Two points A& B are on HP, the point A is 40 mm in front of VP while point B is behind VP. The distance between two projectors is 75 mm. Joining with the straight line their top views makes an angle 450 with XY reference line. Determine the distance of the point B from VP. | 2 | 3 |  | 14 |
| - 8 | a). A line AB 70mm long is inclined 400 HP and parallel to 30 mm in front of VP. The end A is 15 mm above the HP and 20 mm in front of VP, Draw the projections of straight line . b). A line PQ 70mm long is parallel to 40 mm above HP and inclined to VP. The end P is 20 mm, end Q is 50 mm in front of VP respectively . The Draw the projection of the line. Determine the inclination with the VP. | 2 |  | 3 | 7+7 | 3 | 10 |
| - 9 | A line AB of 70 mm long, is inclined 300 to HP and 450 to VP. The end A is 20 mm above the HP 10 mm in front of VP. Draw the projections of the straight line. | 2 | 3 |  | 14 |
| - 10 | A line AB of 70 mm long, has its end A at 15 mm above H.P and 20 mm in front of V.P. Its front view and top view measure 50 mm and 60 mm respectively. Draw the projections of the line and determine its inclinations with H.P. and V.P | 2 | 2 |  | 14 |
| - 11 | The midpoint of a straight line AB is 60mm above HP and 50mm in front of V.P. The line measures 80mm long and inclined at an angle of 300 to H.P and 450 to V.P. Draw its projections | 2 | 1 |  | 14 |
| - 12 | A line AB 120 mm is inclined at 450 to HP and 300 VP. The mid-point C isin VP and 20 mm above the HP. The end A is in third quadrant and end B is in I quadrant. Draw the projections of the straight line. | 2 | 2 |  | 14 |
| - 13 | A regular Pentagonal Plane of 30mm side has one side on the ground. The plane is inclined 450 to the HP and perpendicular to VP. Draw its projections. | 2 | 3 |  | 14 |
| - 14 | Draw the projections of the circle of 50 mm diameter, having it’s plane vertical and inclined at 30° to VP . It’s center is 30mm above the HP and 20mm in front of VP. | 2 | 2 |  | 14 |
|  | Draw the projections of regular Hexagon of each side 30mm having one of the sides in the HP, and its surface makes an angle 450 HP. | 2 | 3 |  | 14 |
| - 15 | A square ABCD of 50 mm side has its corner A in the HP, its diagonal AC is inclined 300 to HP and diagonal BD is inclined 450 to VP and parallel to HP. Draw its projections. | 2 | 2 |  | 14 |