### 23EC4004 – ELECTRONIC MEASUREMENTS AND INSTRUMENTATION

### BRIEF QUESTIONS WITH ANSWERS

### MODULE – 1

PERFORMANCE CHARACTERISTICS OF INSTRUMENTS

- Define accuracy and precision.

Accuracy is the closeness of a measured value to the true or accepted value, while precision is the closeness of repeated measurements to each other. An instrument can be highly precise without being accurate, and a single accurate measurement may still lack precision across multiple trials.

- What is sensitivity in an instrument?

Sensitivity is the ratio of the change in an instrument's output to the corresponding change in the measured input quantity. It indicates the smallest change in the input that the instrument can reliably detect and display.

- What is resolution?

Resolution is the smallest detectable change in the measured value that an instrument can register and respond to. It effectively defines the finest increment of the input quantity that can produce a discernible change in the output reading.

- What is fidelity in measurement?

Fidelity is the degree to which an instrument can reproduce the changes in the measured variable without introducing dynamic error or distortion. High fidelity ensures the output signal is a true representation of the input signal's original shape over time.

- Define lag in dynamic characteristics.

Lag refers to the time delay between the instant an input change occurs and the time the instrument begins to indicate that change. It is a crucial measure of the instrument's sluggishness or slowness in responding to dynamic input signals.

- What causes dynamic error?

Dynamic error is caused by an instrument's inherent physical limitations, such as inertia or capacitance, which prevent it from responding instantaneously to a rapidly changing input. This results in a deviation between the instantaneous true value and the value indicated by the instrument under time-varying conditions.

- What is a DC ammeter?

A DC ammeter is an electrical instrument designed to measure the magnitude of direct current (DC) flowing through a circuit. It must be connected in series with the load and is typically based on the Permanent Magnet Moving Coil (PMMC) mechanism.

- Explain multirange voltmeter.

A multirange voltmeter is a single instrument capable of measuring a wide range of voltages. It achieves this by using multiple selectable series resistors, known as multipliers, to extend the voltage rating of the core meter movement.

- Define ohmmeter. List its types.

An ohmmeter is a device used to measure the electrical resistance of a component or circuit. It contains an internal DC voltage source and a meter movement calibrated to read resistance in ohms (Ω). It has two types : (i)Series Ohmmeter (ii) Shunt Ohmmeter

- What is a multimeter used for?

A multimeter is a versatile electronic instrument that combines the functions of several measuring devices into one unit. Its primary uses are measuring fundamental electrical quantities: voltage (Volts), current (Amperes), and resistance (Ohms).

### MODULE – 2

OSCILLOSCOPES

- What is a CRO?

A CRO, or Cathode Ray Oscilloscope, is a high-speed instrument used to visually display and analyze varying electrical signals as a waveform. It graphs the input voltage on the vertical axis against time on the horizontal axis.

- Define CRT.

The Cathode Ray Tube (CRT) is the vacuum tube at the core of an analog CRO that displays the waveform. It generates, accelerates, and deflects an electron beam onto a phosphor screen to create a visible luminous spot.

- What is the role of vertical amplifier?

The vertical amplifier increases the amplitude of the input signal to a level high enough to cause sufficient deflection on the CRT screen. This stage determines the overall voltage sensitivity of the oscilloscope.

- What is the function of horizontal deflection system?

The horizontal deflection system generates a time-base (sawtooth) voltage and applies it to the horizontal plates to sweep the electron beam across the screen at a constant rate. Its main function is to provide the time axis for the displayed waveform.

- Define sweep trigger.

The sweep trigger circuit synchronizes the start of the horizontal sweep to a precise point on the input signal. This action ensures that repetitive waveforms are displayed stably and do not appear to jump or blur.

- What is a delay line?

A delay line is a circuit element that intentionally delays the signal to the vertical plates after the trigger signal has been detected. This crucial delay ensures that the entire leading edge of the triggered waveform is visible on the screen.

- What are CRO probes?

CRO probes are specialized cables and connectors used to connect the test point in a circuit to the oscilloscope's input. They are designed to faithfully transmit the signal while minimizing the electrical loading effect on the circuit being measured.

- How to measure phase using Lissajous figures?

To measure phase, one signal is applied to the X-input and the second, same-frequency signal to the Y-input, forming an elliptical Lissajous figure. The phase angle ($\phi$) is calculated from the ratio of the ellipse's vertical intercept ($Y_1$) to its maximum vertical deflection ($Y_2$), where $\sin \phi = Y_1 / Y_2$.

- What is a sampling oscilloscope?

A sampling oscilloscope is specialized for measuring extremely high-frequency, repetitive signals that are beyond the bandwidth of conventional scopes. It reconstructs the waveform by taking sequential samples of the input signal over many cycles in equivalent time.

- Define digital frequency counter.

A digital frequency counter is an instrument that accurately measures the frequency of a repetitive electrical signal. It works by counting the number of cycles of the input signal that occur within a highly precise, pre-determined time interval.

### MODULE - 3

BRIDGES

- Define Wheatstone bridge.

The Wheatstone bridge is a four-arm bridge circuit used for the precise measurement of an unknown electrical resistance. When balanced (null condition), the unknown resistance is calculated from the ratios of the three known resistors.

- What is Kelvin’s Bridge used for?

Kelvin's Bridge (or Double Bridge) is specifically designed for the highly accurate measurement of very low resistances (typically below 1 ohm). It minimizes errors caused by the resistance of the connecting leads and contacts, which are significant in low-resistance circuits.

- What is Hay’s Bridge used for?

Hay's Bridge is an AC bridge that also measures unknown inductance, but it is particularly suited for inductors with a high Quality factor (Q > 10). Its bridge balance configuration simplifies the calculation for high-Q coils compared to Maxwell's bridge.

- What does Anderson Bridge measure?

The Anderson Bridge is a highly accurate modification of the Maxwell-Wien bridge used for the measurement of unknown inductance. It uses a standard fixed capacitor and offers high sensitivity for a wide range of inductance values.

- What causes bridge errors?

Bridge errors are caused by physical imperfections like the tolerances of the components, stray capacitance, and stray inductance within the circuit. Environmental factors such as temperature variations and instability of the AC source frequency also introduce inaccuracies.

- State precautions while using bridges.

Precautions include properly shielding all bridge components and connecting leads to minimize the effects of stray electric fields and capacitance. It is also essential to use a highly sensitive null detector and ensure low-resistance contacts for accurate measurements.

- What is balance condition?

The balance condition is achieved in a bridge when the voltage difference across the detector is precisely zero. This results in zero current flow through the detector, allowing the unknown component value to be mathematically calculated from the ratios of the known components.

- What is the purpose of standard capacitor?

A standard capacitor is a high-quality, stable capacitor with a precisely known, certified value. It is used as a highly accurate reference component in AC bridge circuits for the comparison and determination of unknown capacitances or inductances.

- What is stray capacitance?

Stray capacitance is the unintended, parasitic capacitance that exists between any two conductive elements, such as wiring, component leads, or components and ground. It arises from the close physical proximity of conductors and can introduce significant errors in high-frequency AC bridge measurements.

- What is a shielding requirement in bridges?

Shielding requires surrounding the bridge elements with a grounded metallic enclosure to minimize the effects of external electric fields and stray capacitance. This ensures that the measured parameters are not influenced by electrostatic coupling with the environment.

### MODULE - 4

SIGNAL GENERATORS

- Define signal generator.

A signal generator is an electronic instrument that creates and outputs a variety of standard, controllable electrical waveforms (like sine, square, pulse). These signals are used to test, troubleshoot, and characterize the performance of electronic circuits and systems.

- What is AF oscillator?

An AF oscillator is a device that generates continuous, stable alternating current signals within the Audio Frequency (AF) range (20 Hz to 20 kHz). These signals are essential for testing and evaluating audio equipment, speakers, and communication channels.

- What is a function generator?

A function generator is a versatile signal generator capable of producing and outputting multiple repetitive waveforms, including sine, square, and triangular waves. It allows for user adjustment of the signal's frequency, amplitude, and DC offset.

- What is a sweep generator?

A sweep generator is a signal generator whose output frequency is automatically and continuously varied (or "swept") across a defined frequency range. This is used for rapidly testing and displaying the frequency response of filters and amplifiers.

- What is a wave analyzer?

A wave analyzer is a highly selective instrument designed to measure the amplitude of individual frequency components (harmonics) within a complex signal. It uses a narrow, tunable filter to isolate and quantify a specific frequency, primarily for distortion analysis.

- What is a spectrum analyzer?

A spectrum analyzer is an instrument that visualizes a signal in the frequency domain, displaying the amplitude of each component versus its frequency. This is essential for analyzing RF signals, measuring bandwidth, and identifying spurious emissions.

- What is a logic analyzer?

A logic analyzer is an instrument used to capture and display multiple digital signals simultaneously, viewing them as logic states (1s and 0s) versus time. It is a critical tool for debugging and analyzing complex digital circuits, data buses, and microprocessors.

- State standard specifications of signal generators.

Standard specifications include the output frequency range and its corresponding stability over time. Other key specifications are the maximum output amplitude and the purity of the waveform, often quantified by its harmonic distortion.

- What is random noise generator?

A random noise generator is a device that produces a signal with unpredictable, random voltage fluctuations. It generates noise with a uniform power spectral density (white noise) and is used for testing system noise immunity and signal-to-noise ratio.

- What is a frequency sweep?

A frequency sweep is the automatic and continuous variation of a signal generator's output frequency over a set band. This procedure is performed to quickly test how an electronic device responds across a range of frequencies.

### MODULE – 5

SENSORS AND TRANSDUCERS

- What is a transducer?

A transducer is a device that converts energy from one form to another, acting as the interface in measurement systems. It specifically converts a non-electrical physical quantity, like pressure or temperature, into a measurable electrical signal.

- Differentiate between active and passive transducers.

Active transducers are self-generating and produce an electrical signal without an external power source (e.g., thermocouple). Passive transducers require an external excitation source, as the measured quantity only causes a change in their electrical properties (R, L, or C, e.g., strain gauge).

- What is LVDT?

LVDT stands for Linear Variable Differential Transformer, which is an inductive transducer used for the highly accurate measurement of linear displacement. The displacement of an internal core changes the magnetic coupling, producing a differential AC output voltage proportional to the movement.

- What is a strain gauge?

A strain gauge is a passive, resistive transducer used to measure mechanical strain (deformation) on an object's surface. It changes its electrical resistance proportionally when stretched or compressed, and this resistance change is typically measured using a Wheatstone bridge.

- What is a thermistor?

A thermistor is a passive, resistive transducer whose electrical resistance changes significantly and predictably with temperature. They are typically made of semiconductor metal oxides and exhibit high sensitivity to temperature variation.

- What is a thermocouple?

A thermocouple is an active temperature transducer that operates based on the Seebeck effect. It consists of two dissimilar metal conductors joined at one end, generating a voltage proportional to the temperature difference between the junction and a reference point.

- Define RTD.

RTD stands for Resistance Temperature Detector, a passive sensor whose electrical resistance increases linearly with temperature (Positive Temperature Coefficient). Made of pure metals like platinum, RTDs are highly valued for their stability and superior accuracy.

- What is velocity sensor?

A velocity sensor (or velocimeter) is a transducer used to measure the rate of change of an object's position over time, which is its velocity. Examples include electromagnetic devices and tachometers, which convert kinetic energy into a proportional electrical signal.

- What are signal conditioning circuits?

Signal conditioning circuits are electronic stages used to modify the raw output of a transducer before final processing or display. Key functions include amplification, filtering, and linearization to ensure the signal is clean and in the correct format for the measurement system.

- What is an inductive transducer?

An inductive transducer is a passive device whose operation is based on the change of its inductance due to a physical quantity, such as displacement. The change in inductance results from the movement of a magnetic core or a change in the magnetic circuit geometry.