| CourseDetails |  |  |
| --- | --- | --- |
| Class : Third Year, B. Tech Semester: II Year: 2025-26 Course Title: MWOC Course Code: 23A04303T Credits: 3 Program/Dept.: ECE/EVT/ECA Section:A,B,C Batch: 2023-27 Regulation: R-23 Faculty:V.Srilatha, A.vidhyulatha/E.Sowjanya/SK.Asma Sulthana |  |  |
|  | Two Marks Questions with Answers |  |

# Module-1

UNIT – I : WAVEGUIDES

1. What is a waveguide?
        A waveguide is a hollow metallic structure used to guide electromagnetic waves at microwave frequencies. It confines and directs the energy along a desired path. Rectangular and circular waveguides are most common. It works on total internal reflection of EM waves. Used in radar, microwave ovens and satellite systems.

2. Define TE mode.
      TE (Transverse Electric) mode is a mode in which the electric field has no longitudinal component. Ez = 0 in TE mode. Magnetic field has a longitudinal component. TE10 is the dominant mode in rectangular waveguides. It has the lowest cutoff frequency.

3. Define TM mode.
          TM (Transverse Magnetic) mode is a mode where the magnetic field has no longitudinal component. Hz = 0 in TM mode. Electric field has a longitudinal component. TM modes always have higher cutoff frequencies. They are less commonly used than TE modes.

4. What is cutoff frequency?
              Cutoff frequency is the minimum frequency required for wave propagation in a waveguide. Below this frequency, waves get attenuated. It depends on waveguide dimensions and mode. Each mode has a different cutoff frequency. It restricts low-frequency propagation.

5. What are phase and group velocities?
          Phase velocity is the speed of the phase of the wave. Group velocity is the speed of energy propagation. In waveguides, phase velocity is greater than light speed. Group velocity is always less than light speed. They are inversely related.

6. What is meant by power transmission in waveguides?
              Power transmission refers to the transfer of microwave energy through a waveguide. It depends on field strength and characteristic impedance. TE and TM modes carry different power. Losses occur due to wall resistance. Higher modes carry more power.

7. What is attenuation in waveguides?
           Attenuation is the reduction of signal power during propagation. It is mainly due to conductor and dielectric losses. Measured in dB per meter. It increases with frequency. Lower attenuation is desired.

8. What is a circular waveguide?
A circular waveguide has a circular cross-section. It supports both TE and TM modes. It has lower attenuation than rectangular in some cases. Used in satellite communication. Dominant mode is TE11.

9. Give two applications of cavity resonator?
        A cavity resonator is a closed waveguide structure. It stores electromagnetic energy at resonant frequency. It acts like an LC tank circuit at microwave frequencies. Used in oscillators and filters.

10. What is waveguide current?
           Waveguide current refers to surface current induced on waveguide walls. It is caused by alternating electromagnetic fields. These currents cause conductor losses and determine attenuation.

UNIT – II : PASSIVE MICROWAVE DEVICES

1. What are scattering parameters?
        Scattering parameters describe the behavior of microwave networks. They relate incident and reflected waves. Used instead of Z and Y parameters at high frequencies. Measured using a network analyzer.

2. Explain matched termination?
       A termination is a load connected at the end of a microwave line. It prevents reflections of signals. Matched termination ensures maximum power transfer. Short and open terminations cause reflections.

3. Describe the function of attenuator?
          An attenuator is a passive device that reduces signal power. It introduces known loss without distortion. Used to control signal levels. Measured in decibels.

4. Explain the function of phase shifter?
        A phase shifter changes the phase of a microwave signal without altering its amplitude. Used in phased array antennas and communication systems.

5. What is the reason for calling a Tee as a magic tee?

A magic tee is a four-port hybrid microwave junction used for power division and combination. It provides isolation between certain ports.

6. What is a directional coupler?
      A directional coupler couples a small portion of power from the main transmission line. It has four ports. Used for power monitoring.

7. What is a gyrator?
      A gyrator is a non-reciprocal two-port microwave device. It produces a phase shift of 180 degrees and is used in isolators and circulators.

8. Explain the function of isolator?
      An isolator allows microwave energy to pass in one direction only. It protects the source from reflected power. It is a non-reciprocal device.

9. What is a circulator?
      A circulator is a multi-port non-reciprocal device where power flows sequentially from one port to another. Used in transmit-receive systems.

10. How bunching of electrons happens in klystron?

A reflex klystron is a microwave oscillator that uses velocity modulation and a single resonant cavity to generate microwave signals.


                                     UNIT – III : MICROWAVE SEMICONDUCTOR DEVICES

1. Explain negative resistance of Gunn diode?
       A Gunn diode is a transferred-electron microwave device used for generating microwave oscillations. It works on negative resistance.

2. What is IMPATT diode?
       IMPATT diode stands for Impact Ionization Avalanche Transit Time diode. It is used for high-power microwave generation.

3. What is TRAPATT diode?
      TRAPATT diode stands for Trapped Plasma Avalanche Triggered Transit diode. It operates at lower frequencies with high efficiency.

4. Define VSWR?
       VSWR stands for Voltage Standing Wave Ratio. It indicates the degree of impedance matching in a transmission line.

5. Define the importance of Q-factor?
     Q-factor represents the ratio of stored energy to dissipated energy in a resonator. Higher Q indicates lower energy loss.

6. What is microwave power measurement?
         Microwave Power Measurement refers to the process of measuring the amount of power carried by a microwave signal (typically in the frequency range from 1 GHz to 300 GHz).

In microwave systems, ordinary low-frequency methods of power measurement cannot be used because microwaves behave differently (higher frequency, transmission line effects, losses, reflections etc). Therefore, special instruments and techniques are used.

7. What are microwave bench components?
      The microwave bench includes source, isolator, attenuator, waveguide, detector, and VSWR meter.

8. What is impedance measurement?
Impedance measurement determines the resistance and reactance of a load using microwave techniques.

9. What is two-valley model?
It explains Gunn diode operation based on electron transfer between low and high energy valleys.

10. Describe avalanche multiplication?

It is the process of impact ionization that increases carrier concentration in IMPATT diodes. When a semiconductor junction is reverse-biased strongly, the electric field becomes very large.A charge carrier (electron or hole) moving in this field gains very high kinetic energy.


                       UNIT – IV : OPTICAL FIBERS & TRANSMISSION CHARACTERISTICS

1. Define acceptance angle?

Acceptance angle is the maximum angle with respect to the fiber axis at which light can enter an optical fiber and still undergo total internal reflection inside the fiber core.

2. Define and write the formula for numerical aperture?

Numerical Aperture is a measure of the light-gathering ability of an optical fiber. It indicates how much light the fiber can accept from a source.

- Higher NA → fiber can accept light at larger angles

- Lower NA → accepts light only at small angles

3. What is step-index fiber?
       Step-index fiber is an optical fiber that has a constant refractive index in the core and an abrupt decrease in refractive index at the cladding, causing light to propagate mainly by total internal reflection.

Step-index fibers can be two types:

- Single-mode

- Multimode

4. What is graded-index fiber?
        A graded-index fiber is a multimode fiber whose core has a gradually decreasing refractive index from the center to the edge, causing light rays to follow curved paths and reducing modal dispersion.

5. What are types of attenuation in OF?

In Optical Fiber (OF), attenuation means loss of optical power as light travels through the fiber.
The attenuation occurs due to several mechanisms. Main types of attenuation are

1.Material Absorption Loss

2. Scattering Loss

3. Bending Loss

4. Dispersion Loss

6. What is modal dispersion?
Modal dispersion is the pulse broadening in multimode fibers caused by different modes traveling different optical path lengths and therefore reaching the receiver at different times.

# Why Modal Dispersion Occurs

In a multimode fiber:

- Light can travel in many modes (different paths).

- Some rays travel straight through the core (short path).

- Others bounce many times (long path).

- Longer paths → arrive later.

- Shorter paths → arrive earlier.

7. What is chromatic dispersion?
        Chromatic dispersion is the pulse broadening in optical fibers caused by the wavelength-dependent variation in the refractive index of the fiber material and the waveguide structure.

8. Define waveguide dispersion?
         Waveguide dispersion is the spreading of optical pulses caused by the dependence of the mode’s propagation constant on the waveguide structure (core radius, refractive indices, and wavelength).
It arises even if the material of the fiber has no dispersion.

Depends on:

- Core diameter

- Refractive index difference

- Wavelength

9. What is MFD?
      Mode Field Diameter represents the width of light beam in single-mode fibers. Mode Field Diameter (MFD) is the effective width of the optical power distribution in a single-mode optical fiber.
It represents how widely the light spreads inside the fiber core and slightly into the cladding.

10. What is PMD?
       Polarization Mode Dispersion (PMD) is a type of distortion that occurs in optical fiber communication due to different polarization modes of light traveling at slightly different speeds inside the fiber.

## Effect of PMD

- Limits data rate

- Limits transmission distance

- Causes inter-symbol interference (ISI)

- Degrades signal quality

UNIT – V : OPTICAL TRANSMITTERS & RECEIVERS

1. What is the function of optical transmitter?
      A device that converts electrical signals (like data or voice) into light signals, using a light source (LED or laser) to encode information, then launches this light into an optical fiber for high-speed, long-distance communication, acting as the sending end of a fiber optic link.

2. What are the types of LED?
            An LED, or Light-Emitting Diode, is a semiconductor device that produces light when an electric current passes through it, efficiently converting electrical energy into light by recombining electrons and holes in the material. These solid-state lighting sources are known for being highly energy-efficient, long-lasting, and durable, making them popular for general lighting, displays (TVs, phones), 
3. What is the popular basic structure of laser diode?
     A laser diode is a semiconductor device, like an LED, that produces a highly focused, single-color (coherent) light beam through stimulated emission, using a forward-biased p-n junction where electrons and holes recombine to release photons, which then bounce between mirrors to amplify into a powerful laser

4. What is quantum efficiency of optical source?

Quantum efficiency (QE) is a measure of how effectively a device converts photons (light particles) into other particles, such as electrons. It is defined as the ratio of the number of collected charge carriers (like electrons) to the number of incident photons. For example, if 100 photons hit a detector with a 70% QE, it would produce 70 electrons

5. What is PIN photodiode?

A PIN photodiode is a photodetector that converts light into an electrical signal, consisting of a heavily doped p-type layer, an undoped intrinsic layer, and a heavily doped n-type layer. When light hits the intrinsic region, it generates electron-hole pairs, and when a reverse bias voltage is applied, a photocurrent proportional to the light intensity is produced

6. Describe the principle of APD?

APD has several meanings, most commonly Auditory Processing Disorder, a hearing issue where the brain struggles to interpret sounds, especially speech, even if ears work fine, and Acid Peptic Disease, a digestive problem from excess stomach acid damaging the lining (like ulcers/gastritis). .

7. What is detector noise?
       Detector noise refers to random, unwanted fluctuations in a detector's output signal, obscuring the true measurement and limiting performance, especially for weak signals, originating from fundamental physical processes (like quantum/shot noise, thermal noise) and technical issues (like amplifier noise, dark current) within the detector system, and often measured by its statistical properties (like voltage or current variance) to determine system sensitivity.

8. What is optical link design?
           Optical link design is the engineering process of creating a reliable light-based communication path (fiber optic or free-space) by selecting components (transmitter, fiber, receiver) and calculating power/bandwidth budgets to ensure data transmits accurately over a specific distance at a required speed, accounting for losses from splices, connectors, and aging.

9. Explain the link power budget?

A link power budget is a calculation of all the power gains and losses in a communication system to determine if the received signal will be strong enough to be detected reliably. It is crucial for designing telecommunications links, such as optical fiber or wireless systems, by analyzing the transmitted power and all subsequent losses (like attenuation, connectors, and splices) to ensure the received power is above the minimum sensitivity required by the receiver.

10. What is rise time budget?
         A rise-time budget is an optical communication system analysis that allocates available bandwidth among components (transmitter, fiber, receiver) to ensure the total system rise time, calculated as the Root Sum Square (RSS) of individual rise times, meets the required data rate, preventing pulse broadening (Intersymbol Interference) for reliable data transmission. It helps designers confirm the system's speed and manage limitations from dispersion in the fiber.

NECG Faculty                                                    NECG HoD                                              NECG Principal

NECN Faculty                                                    NECN HoD                                              NECN Principal